AUTOMOBIILE INSPECTION FORM

## Driver: _______________________________________ Date: ______________________

Car-Mfg: ____________________________ Model/Year: _________________________

Event: _________________________________________Car #: ____________________

ITEMS    				             Pre Track	ITEMS				          Pre Track

Wheel & Tire Information:           	             Insp Insp	Suspension/Steering:		          Insp Insp

| No cuts/visible defects/tread depth (no cord showing): |  |  |  | Wheel bearing (lift vehicle off the ground, make sure no excessive play or looseness): |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Center/hubcaps removed: |  |  |  | Tie Rods (no excessive play or looseness, bolts tight): |  |  |
| Wheel Condition (no cracks, not bent, etc.): |  |  |  | Steering (no play, no power steering leak, proper alignment, proper fluid level): |  |  |
| Lug nuts tight (typically 90 lb-ft, follow manufacture recommendation): |  |  |  | CV/U-joints (no play, no grease leak, no noise): |  |  |
| Tire pressure (street tires at least 5 psi cold above manufacture recommendation): |  |  |  | Shocks (bolts secure, no leaks, no bouncing): |  |  |

Brakes:							Body:

| Brake pad/shoe linings (at least 50% pad thickness, bed in new pads, no cracked rotors): |  |  |  | Fender to Tire Clearance (no rubbing): |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Brake pedal pressure (firm w/ brake application, not spongy): |  |  |  | Body parts secure: Tow Hooks front and rear (Race Cars Only) |  |  |
| Brake fluid (full level, recommend DOT 4 fluid, recommend new every 2 years, no leaks): |  |  |  | Windshield (no cracks, clean inside & outside): |  |  |

Engine:							Safety Items:

| No fluid leaks (oil, water, fuel): |  |  |  | Battery secured: |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Wires, hoses secured: |  |  |  | Positive Battery Terminal Covered: |  |  |
| Clamps tight: |  |  |  | Helmet SA 2005 M2005 or Newer: |  |  |
| Throttle return springs (returns on its own): |  |  |  | Brake lights (brake lights MUST work): |  |  |

Interior:							 Logbook:  (race cars only)

| Seat secure: |  |  |  | Logbook #: |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Loose items removed (floor mats, etc.): |  |  |  | Issuing Organization: |  |  |
| Safety belts (vehicles passenger same as driver): |  |  |  | Date of Last Annual: |  |  |

I have inspected the above vehicle and take full responsibility for its conditions while participating in this event.

Signed (Driver)___________________________________Date ____________________

OR

I have pre-inspected the above vehicle for the event listed above and find it acceptable for this event.

Signed (Shop Inspector)____________________________ Date ____________________

Name of inspecting shop: ___________________________________________________________

Address:_________________________________________________________________