Washington Viper Club

Autocross Technical inspection Form

This is a self-check and it is your responsibility to preform

It is not necessary to bring this form to the event, by checking the box at time of registration, you are agreeing to preform the check and that you and your car meet the requirements

Do you possess a Valid Driver License and are you at least 18 years of age?	Yes	No

Technical Inspection (to be certified by vehicle owner and/or primary operator):

Exterior:

- Car numbers are not necessary for Viper Autocross Events • No loose or missing body parts, wheel trim rings or hubcaps • All underbody components in good condition and not hanging or dragging

Interior and trunk:

All loose objects removed • Spare tire removed or permanently secured • Seat belts in good condition and operational • Driver’s seat firmly secured • Convertible and/open cars – factory or aftermarket roll bar or hoops preferred • Rear and side-view mirrors in good condition, in place and adjusted properly

Under hood:

- Accelerator return spring operational, if it exists • Battery secure • No fluid leaks • Hood latch secure • All engine accessory belts in place and in good condition •

Suspension and brakes:

- Brake fluid level within specifications • Brake pedal height and firmness within manufacturer specifications • All lug nuts and studs present and torqued to proper specifications • Tire condition (including sidewall) in good condition – no cracks, cuts, patches, or repairs • Wheels in good condition – no cracks, dents or bends that may affect performance or integrity *Brake pads in good condition and rotors not cracked

Driver Preparedness:

- Closed toed shoes • Natural fiber (cotton) or flame resistant clothing • Helmet – Snell rating of SA2000, SA 2005, SA2010 or SA2015 (automotive use rating, no motorcycle helmets) • No loose/hanging clothing (frayed clothing, scarves, etc) • Face shield and/or goggles (for open top cars without factory supplied windscreen)

Please note; you and your vehicle must meet the requirements of this Tech Inspection on the DAY OF THE EVENT or you cannot participate in the event.

I acknowledge that Autocross is inherently dangerous, and that I am solely responsible for my own safety and behavior, agree to follow all published and announced rules including those given in the safety briefing, agree to follow all directions given by course workers or event organizers, certify that I have inspected any car that I may drive in the event, acknowledge that the condition of said car is my responsibility, assert that said car is compliant with all technical requirements, and agree to hold harmless the Port of Shelton, the Washington Viper Club, its officers and directors, event organizers, and event participants from any liability associated with this event.  If I am registering as a Driver, I certify that I am licensed in good standing, and in full compliance with all driver licensing requirements by my local government authority, and that any car I drive meets or exceeds the technical inspection requirements.

Signed:	Date: