CLUB RACING

DRIVER SCHOOL PROGRAM
STUDENT HANDBOOK
[image: image12.png]



Thank you for taking the time to read this handbook.

San Francisco Region, Sports Car Club of America

5250 Highway 162

Willows, CA 95988

530-934-4455
office@sfrscca.org
[image: image1.jpg]Sports Car Club of America




scca San Francisco RegioN
Driver School Program

Goals of the School:
The goal of the school is to graduate students whom we will be comfortable racing against.  We want to stress safety, predictability, consistency, and fun.  Lack of speed rarely, if ever, fails a student from school.  Lack of safety frequently does.  Students need to learn to drive the line, hit apex and exit points more than they need good lap times.  It is much better to see students start slowly, find the line, and then gradually increase their speed, then to see them set good lap times by sheer bravado while unsuccessfully trying to get on the line.  It is very important to get the basics (line, technique) covered first.  This will give a good foundation for further learning.  Students will probably be able to go faster at first by ignoring the basics and simply driving hard, but this will drastically limit potential for further development.  Also, as speeds increase, it is frequently more expensive especially when we go to larger race tracks where there are a lot more solid things to hit at a higher speed!

Grading:
Students are graded on a scale from 1 (failing) to 5 (great).  We grade on a bell curve – that is to say most of the students should average 3’s.  We rarely give 1’s or 5’s.  1 is a failure, 5 would be a truly exceptional student.  In other words, 3 is a good normal grade.  We try to grade in a way that will indicate improvement (or lack of it) as the school progresses.  Also, note that we grade according to how the student should be doing at that particular level of the school.  In other words, an average student showing an average level of improvement would get all 3’s; someone who is totally lost the first session is normal and shouldn’t be degraded for a lack of previous knowledge.
Table of Contents

TECH INSPECTION 3
PADDOCK 4
IN THE PITS 5
ON THE TRACK 6
DRIVING TECHNIQUES 7
FLAGGING & COMMUNICATIONS 11
RULES OF THE ROAD 11
PURPOSE OF F & C WORKERS 14
PHYSICAL LAYOUT 15
RULES WHEN OFF THE ROAD 16
MISCELLANEOUS NOTES 17
BASIC CAR PREPARATION 18
ORGANIZING FOR A RACE 21
RACE WEEKEND ACTIVITIES 23
SUMMARY 25
APPENDIX A 28



Section
1

TECH INSPECTION TC "TECH INSPECTION" \f C \l "1" 
The purpose of tech inspection is to be certain that all cars comply with the required safety regulations.  Tech will also check that the car is properly prepared to be on a race track.

It is not the responsibility of tech inspection to check non-safety class rules at pre-race tech.

TECH INSPECTION AT Thunderhill Raceway Park IS HELD AT North End of the Paddock.
1. Your car and driver equipment must be teched before your car enters the course.

a. In the event that your car is involved in an accident on the track or suffers any significant damage from going off course, the car must be re-teched before going out in the next session.

2. Have your tech sheet and log book filled in and signed before arriving at tech.

3. All drivers' gear must be ready for inspection:

a. Drivers' suit and underwear (Nomex or approved fire-retardant material).  A Nomex hood is also required if the driver has facial hair or other hair that protrudes from beneath a driver’s helmet.

b. Helmet – Must meet the requirements of GCR Section 9.3.19.C.2; The back of the driver helmet will be labeled with a minimum of the driver’s name and the use of a head and neck support system is highly recommended.

c. Gloves - Must be Nomex or leather with NO vent holes.

d. Shoes - Must be Nomex or leather and used with Nomex socks.
4. After successfully passing Tech Inspection, you will receive a tech sticker.  This sticker should be placed Left Side Rollbar.  

Section
2

PADDOCK TC "PADDOCK" \f C \l "1" 
The paddock is defined as the area used for parking race cars and support vehicles and for working on the race cars.  This area also includes the timing tower and false grid areas.

1. No race car engine may be operated prior to 7 AM or after 6 PM on any day.

2. Each driver is allotted paddock space for his race car and trailer. 
3. The paddock access road will be closed to pedestrian traffic while race cars are on the track.  This includes practice, qualifying, and race sessions.  Exceptions may be made by the Chief Steward.
4. Children under the age of 12 are allowed in the paddock area only if they are strictly supervised by an adult.  Failure to do so may result in expulsion from the paddock area.
5. Consumption of alcoholic beverages, drugs and similar controlled substances are prohibited in paddock, pits, and infield areas during the hours of competition. Illegal substances are NOT allowed at any time.

6. No driver, crew member, worker, or official who has consumed any alcoholic beverage or drugs on the day of the event will be allowed to participate.  Violations of this regulation will automatically result in immediate ejection from the paddock area.  Drivers will, in addition, be disqualified from the event and will be required to appear before the Stewards of the Meet to justify why they should not be suspended from competition for a period of 12 months.
7. Drivers are at all times responsible for the conduct of their crews.

8. Riding on the exterior coachwork of any vehicle in the paddock area is prohibited. 
9. Pets are permitted only if leashed or otherwise restrained in a suitable enclosure. 
10. Reckless driving in the paddock may result in immediate expulsion from the event and/or grounds.
Section
3
IN THE PITS TC "IN THE PITS" \f C \l "1" 
The pit area is defined as the pit lane that runs parallel to the front straight.
1. The pit area is the only area on the race track other than the paddock that allows crew people within the track limits.
2. No driver may have more than 3 attendants in the pits.  This number may be decreased at the discretion of the Chief Steward.
3. Minors, fifteen (15) to seventeen (17) years of age, may be issued pit credentials only if they hold the proper minor crew license.  All other persons under eighteen (18) years old are prohibited from entering the pit area or any other hazardous area.
4. Unless a car is physically in the pit, only one attendant is permitted in the pit lane.
5. No smoking is permitted in the pit lane.

6. Pit crews are, at all times, under the control of the Pit Stewards.

7. A vehicle which is no longer able to compete must be removed from the pit lane as soon as practical.

8. A vehicle which leaves the track/pit lane and enters the paddock area during the course of a race may not re-enter the competition, unless with the approval of the Chief Steward.

9. The GCR does not require Pit Crew members to wear long pants, shirt, and shoes in the pit lane, however, it is suggested that appropriate attire be worn.  The supplemental regulations, as in this Driver's School, may require long pants and closed-toe shoes.  Be sure to check the published supplemental regulations.
Section
4

ON THE TRACK TC "ON THE TRACK" \f C \l "1" 
1. DRIVERS MEETINGS:  All drivers must attend all drivers' meetings.  They are held at Thunderhill Club House First Floor and Second Floor.

2. NO VEHICLE is allowed on the track at any time without permission of the Chief Steward.
3. PASSING:  When a pass is being attempted, the passing driver has the responsibility of making the decision of when to pass.  The overtaken driver has the responsibility of not blocking and watching his mirrors so that he does not run into the passing driver when the move is actually made.

4. HAND SIGNALS:  Each driver has the responsibility of making proper, visible hand signals to inform drivers behind you and to convey to workers when problems arise.  These situations specifically include, but are not limited to, the following:

a. When entering pit road from the race track

b. When moving slowly or stalled on the track

c. In the event of a "no start"

d. When being passed by a faster car, signal to him where you expect him to go

e. When you have left the track, signal to the turn workers when you are ready to proceed and then await the signal to go

5. Unless it is an emergency, always exit the track via the pit lane.

6. If your car has a problem out on the track and cannot return to the pits, do as the turn marshals say, and leave your car where they tell you.

7. In an emergency situation requiring the use of an emergency vehicle on the course, a white flag will be displayed standing for two (2) flag stations prior to the vehicle.

8. In the event a red flag situation arises, all competitors will stop racing, come to an immediate, controlled stop at the side of the race track and wait for instructions.  When released by an Official, proceed cautiously to the pits.

Section
5
DRIVING TECHNIQUES TC "DRIVING TECHNIQUES" \f C \l "1" 
1. Maps of Thunderhill Raceway Park can be found on the following link.  Thunderhill Track Maps Keep in mind for Drivers School we will use the 1.9 Mile track on the eastside. Learn the traditional line first.  You have to know what you are doing before you can modify it.  After becoming comfortable with the traditional line, individual drivers might adapt or modify the line slightly to suit their particular car’s handling characteristics.

2. You must maintain SMOOTH control to get maximum speed.  Your entry into a turn (turn-in point) must be performed smoothly, never jerk the steering wheel abruptly.  Time your turn-in point so that the car hits the apex of the turn without having to readjust the car’s attitude.

3. The driver should establish landmarks on the race track to help remember braking points, shift points, turn-in points, and turn apex points.  These landmarks could be speed bumps, a damaged piece of pavement, a turn station, etc.  Whatever you choose be certain that it can't move.

4. "EARLY APEX" or "LATE APEX" refers to adjusting the ideal apex of a certain turn to account for interaction with an upcoming​ turn, setting yourself up to pass another car, or correcting a handling characteristic of your particular car.  Use a late apex where appropriate (especially high-powered cars).

5. Keep hands at 3:00 and 9:00 o'clock.

6. Brake hard and in a straight line.

7. If possible, do not change gears in a turn.

[image: image2.png]1 to 2 inches Brake Pedal
! Applied Position .ee e

Free Position - Accelerator



[image: image3.jpg]


[image: image4.jpg]



[image: image5.jpg]


Driving Techniques continued...

[image: image6.png]



[image: image7.jpg]


[image: image8.jpg]


[image: image9.png]


[image: image10.png]


[image: image11.png]




Driving Techniques continued...

8. "HEEL AND TOE" is a method of downshifting smoothly while braking.  If performed properly, the engine speed will match the vehicle speed in the lower gear as the clutch is engaged.

HEEL TO TOE SHIFTING



a. Most drivers use the ball and outside edge of the right foot, not the heel and toe, to downshift while braking.  To do this easily the brake and accelerator pedals must be positioned so the brake pedal ends up slightly above the accelerator when the brakes are applied.  The lateral distance between the pedals should be 1-2 inches, as shown.

b. Brake with the ball of the right foot on the edge of the brake pedal and bout half of the outside edge of the right foot over the accelerator pedal as shown.  Do not lift the heel off the floor.  When the car has slowed enough to engage the next lower gear without over-revving the engine, perform the following as quickly and smoothly as possible:

c. Disengage the clutch. Roll the edge of the right foot onto the accelerator while keeping a constant force on the brake pedal (this is the hard part).

d. While the engine is accelerating, engage the desired gear.

e. When the engine speed matches the transmission input shaft speed, roll the foot off of the accelerator and engage the clutch (this is hard, too).  When this matching is done properly the car will not lurch forward and the drive wheels will not lock up.

f. While learning this procedure go down one gear at a time (4th, to 3rd, to 2nd, etc.)  When the procedure is mastered try skipping gears.  This saves time and wear on the engine and driveline, but the matching process is more difficult.

g. Heel-to-toe down shifting is very important and it is not easily learned.  If possible, practice in a 4 or 5 speed street machine.  Most road cars require accelerator pedal position adjustment for easy heel and toe shifting.  A piece of wood bolted to the existing pedal works fine.

Driving Techniques continued...

9. CAR HANDLING – Depending on how your car is set up, you will experience on of the following handling characteristics:

a. Understeer is the tendency of the car to turn less than the front tires are telling it to turn.  Also called plow or push.

b. Oversteer is the tendency of the car to turn more than the front tires are telling it to turn.  Also called loose.

c. Neutral Steer is when the car responds to the front tires as expected.

The following chart may be useful if you want to modify or improve your car’s handling characteristics:

	ADJUSTMENT
	INCREASE UNDERSTEER
	INCREASE OVERSTEER

	Tire

Pressure
	Front
	Decrease
	Increase

	
	Rear
	Increase
	Decrease

	Tire

Section
	Front
	Smaller
	Larger

	
	Rear
	Larger
	Smaller

	Wheel

Camber
	Front
	More Positive
	More Negative

	
	Rear
	More Negative
	More Positive

	Springs
	Front
	Stiffer
	Softer

	
	Rear
	Softer
	Stiffer

	Sway

Bar
	Front
	Thicker (Stiffer)
	Thinner (Softer)

	
	Rear
	Thinner (Softer)
	Thicker (Stiffer)

	Weight Distribution
	Move Forward
	Move towards Rear


*Increasing Toe-in will generally increase understeer (to an extent), but at the sacrifice of straight-line speed.

Section

6

FLAGGING & COMMUNICATIONS TC "FLAGGING & COMMUNICATIONS" \f C \l "1"  
1.  RULES OF THE ROAD TC "RULES OF THE ROAD" \f C \l "2"  - Communications between Drivers and F&C Workers

a.
Flags - At all turn stations and Starter’s Stand

     





GREEN (Solid Green):  





A race is under way the instant the green flag is displayed.  This flag  




shall normally be in possession of the Starter only and shall not 




ordinarily be displayed at the flag stations around the course.  When 




displayed, the green flag indicates that the course is clear.





STANDING YELLOW (Solid Yellow):  


Take care, Danger, Slow Down, NO PASSING between the flag and 

Emergency Area.

  


WAVING YELLOW (Solid Yellow):  
       



Great Danger, Slow Down, be prepared to stop — NO 





PASSING between the flag and Emergency Area





DOUBLE WAVING YELLOW (Solid Yellow):  




Great Danger, Slow Down, be prepared to stop — FULL COURSE 




IS WAVING YELLOW — NO PASSING ANYWHERE ON 




THE COURSE.  Shall be used with or without a pace (safety) car, 




including pace lap(s).


Flagging and Communications cont...
Cars may carefully pass emergency vehicles.  Cars may also pass other cars that are disabled and cannot keep the pace as signified by a raised arm on the part of the driver of the disabled car. NOTE:  A driver may encounter several flags before reaching the emergency area.  The requirements are still the same “SLOW DOWN, NO PASSING.”



BLUE FLAG (Blue with Diagonal Yellow Stripe):  


Another competitor is following you very closely or is trying to 


overtake you.  This flag may be displayed standing or waving, 



depending upon the speed with which you are being overtaken.


SURFACE CONDITION FLAG (Yellow with Vertical Red 
Stripes):  



Take care.  Oil has been spilled or a slippery condition exists, or 
debris is present on the course.  This flag is displayed standing.


WHITE FLAG (Solid White):  




An ambulance, service vehicle, or slow moving (e.g., with mechanical 


trouble) race car is on the circuit.  Take care.  This flag shall be 


shown standing for two (2) flag stations prior to the vehicle.  A 


standing white flag shall also be displayed during the first lap of a 


practice or qualifying session to indicate the location of the flagging 


stations.


BLACK FLAG (Solid Black) 



CLOSED BLACK FLAG (Furled):  


WARNING — you are driving in an unsafe or improper manner.  If 


continued, you shall be given the Open Black Flag.  



OPEN BLACK FLAG (Standing):  
Complete the lap you are now on, and then stop for consultation at the location designated by the Chief Steward or the Supplementary Regulations for that event. OPEN BLACK FLAG with the word "ALL" displayed:  All cars proceed directly to the pits.  Restarts are the same as for a red flag.


Flagging and Communications cont...



MECHANICAL BLACK FLAG (Black with Orange Ball):  
There is something mechanically wrong with your car.  Proceed to your pit at reduced speed.
CHECKERED FLAG (Black and White Checks):  
You have finished the race (or practice/qualifying session).  Continue cautiously to pits.

RED (Solid Red):  
Come to an immediate, controlled stop, taking care not to endanger you or another car by the manner of stopping.  Pull the car to the edge of the track to the extent circumstances permit. THE SESSION HAS BEEN STOPPED.
NOTE: Red flag can only be ordered by chief steward.
LIGHTS INSTEAD OF FLAGS:  The Supplementary Regulations shall state where on the course and for what purpose lights shall be used.
STOPPING A COMPETITION:  When it is necessary to stop a competition, the Chief Steward may:
· Order a black flag and an “ALL” sign to be displayed on the Starter’s stand; simultaneously, each flag station around the course shall display a black flag.  These black flags shall inform all drivers that they shall stop racing immediately and proceed to their pits, exercising extreme caution and being prepared to stop if necessary.

· Order a red flag to be displayed simultaneously at all flag stations.  Further instructions shall be conveyed by the Corner Officials.  Once a red flag has been displayed, it shall not be withdrawn until all cars have come to a stop.

· Order the Checkered Flag to be displayed to the lead car if fifty (50) percent or more of its scheduled time or distance has been completed.


Flagging and Communications cont...

b.
Worker's Hand Signals to Drivers - Given along with flags:
i. Go Easy!:  Both palms down, hand held stretched in front, repeated up and down motion with arms.  Used when there is severe track blockage ahead or conditions require you to reduce speed.

ii. Pull Towards Worker:  Palms toward worker, scooping motion with hands towards worker.  Used when only the worker's side of the track is clear.

iii. Push Away From Worker:  Palms away from worker, pushing motion away from worker with arms.  Used when only side of track opposite worker is clear.

iv. Slippery Track Conditions:  One arm palm down, moved in a circular motion parallel to the track.  When there is an oil or coolant spill on the track.

v. Point Procedure:  When driver requests a point, the worker holds out a hand as a stop signal, and then makes large, full arm pointing motion at the correct car to pull out onto the track behind.

vi. Fire Extinguisher Held Overhead:  You are on fire!  Pull into turn station if workers are pointing at you.  We can then put out your car!

c.
Driver's Signals to Workers
i. Engine Running, Want a Point:  One arm raised, tells worker engine is still running.

ii. Engine Dead, Can't Start:  Both arms raised, driver not trying to start car.

iii. Acknowledge A Flag:  Nod head, check your mirrors for a blue flag, or wave with one hand (black flags & meatballs only).  We'll be looking closely for any signal.

iv. Dummy Ahead!  Point at car ahead leaking oil, blocking, etc.  We'll keep an eye on him.

Flagging and Communications cont...
2.
PURPOSE OF F & C WORKERS TC "PURPOSE OF F & C WORKERS" \f C \l "2" 
a.
Communications with Drivers
i. Flags:  Painted paddles, MODIDs (Manually Operated Driver's Information Device) are still used at some tracks.  Extra large yellow flags are used at some especially treacherous turns.

ii. Lights:  Normally located at tunnel entrances, has same meaning as flags.

iii. Hand Signals:  In addition to flags.

b.
Communications with Stewards - Through the Control Network.

i. Course Condition:  Incidents, oil spills, emergencies, etc.

ii. Competing Cars:  Mechanical trouble, metal to metal contact, etc.

iii. Any Other Situation:  Crowd control, etc.

c.
Relaying Steward's Orders
i. Emergency Services:  Steward will order emergency services onto track through corner workers on many tracks.

ii. Drivers:  Black flags, furled blacks, meatballs, waving blue flags, etc.

iii. Turn Workers:  Through sign language.

d.
Safety - We are the first line of safety.  Protect in this order of priority:

i. Drivers
ii. Spectators
iii. Other workers
iv. Cars
v. Other property

e.
Maintaining a Clear Course ... a prime objective.  The reason why you can't work on your car for 27 laps in the impact area.  This is a good reason to pull off at the turn station where the car is in a safe area.

Flagging and Communications cont...

3.
PHYSICAL LAYOUT TC "PHYSICAL LAYOUT" \f C \l "2" 
a.
Chiefs Of Flagging And Communications At Control - Communicates to stewards and to all turn stations.

b.
Corner Stations:
i. Sufficient number to cover track.  Must have complete visibility of the track.

ii. Located for visibility, generally inside of turn.

iii. Manned with sufficient workers for common incidents.  Minimum of 3 with more at high incident turns.

iv. Communications, safety, and maintenance equipment.  Makes turn station best location if you need immediate help.

· Phone communications

· Signal flags, lights, or paddles

· Fire extinguishers

· Pry bars, brooms, oil dry, etc., plus usually our own tools

4.
RULES WHEN OFF THE ROAD TC "RULES WHEN OFF THE ROAD" \f C \l "2" 

a. Off Track Excursions
i. Still Moving:  Simple 4 wheels off, continue, re-merge into traffic.

ii. At A Stop:  Give one arm or two arm signals to workers.  We don't like running with 20 lb. fire bottles for 200 yards when we don't have to!

b. Loss Of Power Or Mechanical Failure
i. Workers in Charge:  You own the black stuff; we own the green stuff.  Our job is to get you back on track if it can be done safely and timely.  Pit crew is not to come out and help during a session.

ii. Can Work On Car If Safe:  Safe is either 300 yards away from track, or at the turn station itself, behind the Armco.  We prefer the latter; we also often have tools at the turn station.


Flagging and Communications cont...

c.
Serious Incident
i. Turn Workers First Response:  Always with a fire bottle.

ii. Emergency Services Responds If Required:  In case of injury potential.

iii. Emergency Services or Worker in Charge:  If Emergency Services is called, they are in charge of the incident.  If not called, the turn captain is in charge.

iv. If Sent To Medical - Go!  Sent after serious impacts, rollovers, etc.  Please don't throw your helmet either; we have to call that in and it will cost you an X-ray!

d.
Rope (Flat) Tow Procedure
i. Helmet:  Tow ropes break.

ii. Gloves:  Prevent rope burns

iii. Lap Belt:  Brakes that did work suddenly don't, or car may roll over!

iv. Tell Service Crew Of Any Problems & Pit Location:  If you can't steer, or the brakes don't work, let them know before you proceed under tow.

e.
Wrecker/Tilt Bed Procedure
i. Driver with Car If Possible:  To tell wrecker crew pit location and how-to pick-up car.

5.
MISCELLANEOUS NOTES TC "MISCELLANEOUS NOTES" \f C \l "2" 
a.
Common Courtesy:  i.e., if you are stuck in the middle of the track with a dead engine and we come to push you off the track with a horde of cars chasing us, don't drop the clutch and make it a "surprise" push start!

b.
Common Sense:  "To finish first, first you must finish."  Not slowing down for a waving yellow with a hazard can often put you in the weeds with the first 8 cars to ignore the flags!

Flagging and Communications cont...

c.
Other Reasons to Visit a Turn
i. Learn from other drivers
ii. Learn how corner crew operates
iii. Postpone loading a broken race car
iv. You might even give us a better perspective

Section

7

BASIC CAR PREPARATION TC "BASIC CAR PREPARATION" \f C \l "1" 
*** Have Car Ready to Race before Coming to School ***

1. Engine Compartment

a. Clean
i. Degrease before each race

ii. Light color paint helps trace oil leaks

b. Battery

i. Positive (hot) post covered

ii. Securely anchored

c. Overflow Tanks

i. As required/necessary

ii. Proper capacity

iii. Securely anchored

d. Wires, Hoses, etc.

i. Securely anchored

ii. Away from belts, fans, pulleys, headers, etc.

2. Engine

a. Clean

b. Use mild performance engine!

i. Handling, not horsepower, works at Driver's School

ii. Must be reliable enough to last through school

c. Sound Control Requirements. Mufflers required. See Region Supplemental Rules for sound limit. 
d. Select proper gears for track.

Basic Car Preparation cont...
3. Suspension

a. Clean

i. Light colored paint can help locate cracks or failures.  Not 100% accurate, but very helpful.

b. All parts tight

i. Inspect before each race weekend, before each race.

c. Front end must be properly aligned before anything else will work properly.  Rear end also, where applicable.

d. Brakes

i. Top quality materials

ii. Adjust bias (where possible) to lock front brakes slightly before rears.

iii. Change brake fluid frequently.  The higher the DOT number the better it is, but the faster it can deteriorate by absorbing moisture.

e. Wheels

i. Inspect frequently.  Paint can help detect cracks.

ii. Don't over torque lug nuts.  Use manufacturer specs.

f. Check for the clearance and suspension bottoming.

g. If you are running racing slicks, also bring rain race tires or good street radials in case of rain.

4. Drivers Compartment

a. Clean

i. Vacuum, remove all debris before race

b. Seating position critical.  Must be able to see/reach:

i. Steering wheel.  Good grip?

ii. Kill switch and other controls within easy reach

iii. Gauges ... must be able to see at a glance

iv. Mirrors ... close to the eyes and vibration proof

v. The road ahead (sounds silly but often overlooked)

vi. Shift lever ... be able to reach comfortably

vii. Pedals ... must be able to heel and toe

c. Rollbar/Cage must be above head and padded per GCR.

d. Harnesses must be securely installed per GCR.
Basic Car Preparation cont...
e. Fire extinguisher/equipment must be securely mounted per GCR.
f. Recommend sedans leave defroster ducts and blower intact.
5. Body Work

a. Clean

b. Must be straight and painted, not primed.

c. Numbers and letters contrasting color and located and sized per GCR.

d. Window retaining clips/straps per GCR.

e. Taped lights (where applicable) per GCR.

f. Window net per GCR.

6. Driver Preparation
a. Get into best physical and mental condition.

b. Get sufficient sleep before school.

c. Select a drivers' suit that is loose enough for unhindered movement.

d. Bring sunglasses or a tinted helmet shield.

e. Study GCR before school!  Learn the meaning of the flags well enough that you will react instinctively.

Section

8

ORGANIZING FOR A RACE TC "ORGANIZING FOR A RACE" \f C l "1" 
1. The better organized you are before a race weekend, the less work and worry you will have during the event.  Develop a checklist which fits your needs.  Try to have everything packed, loaded, and checked off by Friday afternoon.

2. Obtain a good crew for car maintenance and timing to allow you to concentrate on driving.

3. Registration Materials

a. Briefcase or folder with membership card and Novice Permit or License

b. Checkbook/cash
4. Drivers Equipment

a. Gear including helmet, suit, underwear, socks, hood, shoes, and gloves
5. Race Records

a. Stop watch

b. Clipboard/notebooks
i. Lap charts

ii. Mechanical record (fuel/oil consumption, chassis settings and changes, repairs, and replacements)

iii. Pit board
6. Tools

a. Possession of a basic mechanic’s tool box is assumed.
b. Additional items which may come in handy:  Feeler gauge, files, can opener, chalk, oil filter wrench, tire gauge, drill and drill bits, wire wheel, emery paper, torque wrench ("click" type is best, preset to wheel nut specs), magnet, scissors, and knife
Organizing for a Race cont...
2. Support Materials

a. Equipment:  Hydraulic floor jack, jack stands (don't skimp on quality), several 18" squares of thick plywood (4), gas can with spout, water bucket, drain pan (do not dump oil, antifreeze, etc. on the ground ... a waste tank is available).

b. Materials:  Oil change plus one quart, spark plugs, brake fluid, duct tape, RTV silicone, rags, paper towels, hose clamps, primary wire, penetrating oil, gear lube, Windex, engine degreaser, antifreeze, gasket material, funnel, neoprene hose, mechanics wire, electrical tape.

c. Parts:  Belts, bearings, hoses, lug nuts, lug bolts, seals, brakes, assortment of bolts, nuts, washers, screws, cotter pins, radiator cap, oil filter, thermostat and gasket, brake light bulbs, distributor cap and rotor.

Section

9

RACE WEEKEND ACTIVITIES TC "RACE WEEKEND ACTIVITIES" \f C \l "1" 
1. Registration
a. Be on time
b. Have all materials ready
c. Have all crew names on crew list

2. Tech Inspection
a. Find and mark a paddock spot
b. Unload car/equipment
c. Remove trailer to trailer parking area immediately
d. Push car (before 10:00 AM) to tech area
e. One person must be with car at all times during tech
f. Have all materials ready including logbook filled out, tech sheet filled out, all personal safety equipment, car clean and emptied of tools, etc.

3. Track Sessions
a. Be prepared when session/race is called
b. Tire pressures
c. Fuel
d. Safety equipment on
e. Car ready (warmed up, wheels tight, etc.)
f. Support materials to grid

Race Weekend Activities cont...

4. Driver Preparation
a. Pre-Weekend

i. Avoid fatigue
ii. No alcohol consumption within 24 hours of weekend
iii. No medication other than normally prescribed (use doctor's advice/own judgment)
iv. Be sure car is properly prepared

b. Pre-Race

i. Empty your bladder

ii. Avoid large meal or large amount of fluid immediately prior to driving. However, continue normal intake of fluid during the day.

iii. Empty mouth of gum, candy, dentures, etc.

iv. Empty pockets

v. Empty car cockpit of loose tools, debris, etc.

vi. Remove jewelry, watches, etc.  However, it is advisable to wear a "medic alert" type bracelet in addition to information on helmet.

vii. ABSOLUTELY NO SALT TABLETS.  Eat salty foods and consume adequate liquids on very hot days.

viii. Wear all safety clothing

ix. Wear all belts in proper location

x. DO NOT RACE if you feel upset, ill, or otherwise "out of sorts".
c. Race/Post Accident

i. If accident is inevitable and if you can:
· Grasp steering wheel at bottom with both hands

· Move left foot back by seat, away from pedals
· Duck
· Count to 10 before looking up.  It's amazing how long a car can stay in the air before finally coming to rest.
ii. If injured, remain still

iii. If uninjured, wave at the nice turn workers

iv. Leave helmet on

v. Move away from car to safe area, if able

vi. Follow directions of turn workers at all times

vii. Rollovers require you to ride back to the pits in the ambulance

viii. If a fuel leak or fire is present, get out and roll over the ground.  Keep rolling.  This will help put out any fire and bring the turn worker's attention to the fact that you may be on fire.

Section

10

SUMMARY TC "SUMMARY" \f C \l "1" 
1. Rules Of The Road:  We are here to be safe and have fun. This is Club racing, not F1, IRL or NASCAR.  Take care of each other out on the track.  This does not mean we can’t be competitive.  It means you have to leave the other driver enough room to survive.  While it is primarily the responsibility of the overtaking driver to make a safe pass this does not remove all responsibility from the driver being overtaken.  The GCR states:
Section 5.6

A.  It is the responsibility of all drivers to avoid physical contact between cars on the race track.
B. All competitors have a right to “racing room” on the marked racing surface.  “Racing room” shall be generally defined as sufficient space on the marked racing surface so as to allow a competitor to maintain control of his car in close quarters, under racing conditions. 
C. It shall be incumbent on all drivers to preserve the right of his fellow competitors to “racing room” on the race track.  Abrupt changes in direction so as to impede or affect the path of a car attempting to overtake or pass may be interpreted by Officials as an attempt to deprive a fellow competitor of his right to “racing room”.

2. Flags:   We can not over emphasize the importance of flag recognition and adherence.  We realize that the students are in an unfamiliar, stressful environment and probably struggling to keep up with what they’re doing inside the car, but there simply are NO valid excuses for missing flags.  The workers spend their time and money so we can play. The least we can do is respect them enough to pay attention to what they are trying to tell us.  You may feel safe inside your roll cage, but that flag may be for an unprotected worker who is trying to save the life of one of your friends.  Winning a race is simply not worth risking their life.

** Always pay attention to where the flag stations are on your warm-up lap. A wave or nod to the workers might just get you a new friend or two.

3. General Driving Practices:  Think! Use warm-up laps to check track conditions such as water or oil on the surface, pavement breaking up, condition of dirt in run-off areas, and the location of manned worker stations.  Also, get the feel of your car and get your oil, tires, and brakes up to temperature.  Use cool-down laps to gradually cool your tires and brakes, drive the line, and thank the workers.  Be smooth.  Jerky or abrupt motions will upset the car.  It IS possible to move quickly AND smoothly.  Relax! You’ll be in a high stress environment, but the more relaxed you are the better you’ll be able to feel what the car is doing and the smoother your motions will be.  A death grip on the wheel will make your motions jerky and prevent you from feeling the subtle messages your front tires are telling you.  Try to be consistent. Make small changes.  Don’t repeat mistakes.  If you have a problem in a particular corner, don’t just keep doing the same wrong things.  Try something different.  If you don’t know what to try, just slow down until you can stay on the line, then gradually increase your speed at a rate that lets you stay on line.  Have escape routes planned.  Know where it is safest to go off the road.  
“If I spin in this corner, do I want to spin to the inside or outside?  
Can I safely drop wheels off on the exit of this corner?”
It’s best to know before you need to know.  Have planned contingencies if you make a mistake such as missing a braking point or turn-in point or an apex.  The first rule is to survive the corner.  Don’t worry about trying to get back on line or how fast you’re going or how early you can get on the gas.  Just survive and figure out what you did wrong and try not to repeat it next time! 

4. Getting Up To Speed:  Think ahead.  The faster you go the farther ahead you need to plan.  Focus on the next event i.e., when you are at the braking zone look for your turn in point, at the turn in point look at the apex, etc.  If things are coming at you before you’re ready for them, you either need to focus farther ahead or slow down until you can stay mentally ahead of the car. Always focus ahead trying to figure out what you did wrong in that last corner is a good recipe for going off in the next one.  Get comfortable with driving right to the edge of the track, and with being a couple of inches away from other cars.  Highway driving habits can be tough to break in this regard, but you need to do it.  Remember, that on most cars corner exit speed is more important that entry speed, especially on corners leading onto long straights.  Sacrificing some entry speed to be able to get on the gas sooner usually gives better lap times.  Learn to keep your tires at their limit of adhesion as much of the time as possible.  Practice threshold braking.  Find a comfortable driving position where you’re not hanging on to the steering wheel to stay in the seat.  Hands should be at about 3 and 9 o’clock and you should pull rather than push to turn the wheel.  Make sure you are comfortable with heel and toe downshifting.  Practice in your street car (if it has a manual transmission) until it becomes second nature.

5. Starts And Pace Car Situations:  OBEY THE YELLOW FLAGS!  Raise your hand (and give a wave) when slowing on the track to alert drivers behind you.  Remember to do this any time you are slowing down on the track for flags, mechanical problems, or when you are entering the pits.  For SCCA racing when the green flag comes out on a start or race restart, the entire track goes green and you can start passing, no matter where you are on the track (unless there is a local yellow flag where you are).  After a pace car period, sometimes the entire pack won’t catch the pace car before you go green.  Watch the flaggers and when you see them drop the yellows –GO!  Also, during pace car periods you should try to catch up with the pack.  Don’t overdrive.  Wherever the situation is, you should leave plenty of space for disabled cars, emergency vehicles, or workers, but the rest of the track you should go relatively quickly until you catch the pack.

6. Dealing with Traffic:  Learn to set up your passes.  Think ahead.  See where you are quicker than the other car and that is where you should try to pass.  See rules of the road above.  When racing remember that two cars passing each other frequently slows both cars (side by side, neither car can use the whole road).  This means when someone does the banzai late braking move to pass you often it will slow them down in the corner enough that if you change your line to maximize your exit speed, you can often re-pass them on the following straight.  When being overtaken by faster cars point them by.  Always point where you want the other car to go but remember that this is advisory.  He may already be committed to doing something else.  It does let the other driver know that you see him though.

7. Putting It All Together:  As you get up to speed you will experience slip angles.  In other words, your car will not be traveling in the exact same direction that your tires are pointed.  You must learn to compensate for this. Simply aiming for the apex may get you there at slow speeds but as speeds increase the slip angles will increase and you will miss the apex unless you aim inside the apex by the amount you would have missed it by.  Driving a race car is an athletic process.  In addition to learning the rules and techniques it requires practice.  There is no substitute for quality track time.  Practice, practice, practice.  At the end of the day SCCA has a social gathering (free beer, sodas, munchies) for all participants.  Come on by and meet your fellow competitors and thank the workers.  Have fun …….

APPENDIX A TC "APPENDIX A" \f C \l "1" 
FLAG CHART
The following chart may be helpful in remembering the flags and their meanings:

	Flag 
Color
	Additional Conditions
	Flag Location
	Flag Meaning
	Driver Action Required

	Green
	
	Starter’s Stand
	Track is clear
	Race normally

	Yellow
	Stationary
	All Stations
	Incident off track — possible danger
	Take care — No passing

	Yellow
	Waving
	All Stations
	Incident on track — 
great danger
	Slow down — No passing — Prepare for evasive action

	Double Yellow
	Waving
	All Stations
	All corner stations are under a waving yellow condition
	Slow down — 
No passing anywhere on course — Prepare for evasive action — Safety Car may be on course

	Red
	
	Starter’s Stand -

All Corners
	Race is stopped
	Stop racing immediately — Controlled stop on track

	Black w/"All"
	Waving
	Starter’s Stand -
All Corners
	Race is stopped
	Stop racing immediately — Proceed to pits

	Blue w/stripes
	
	All Stations
	Driver close behind you
	Check mirrors

	Blue w/stripes
	Waved or shaken
	All Stations
	Driver behind you is attempting to pass
	Check mirrors - do not block

	Yellow w/Red stripes
	
	All Stations
	Oil, water, or debris on track — 
slippery conditions
	Slow down — 
Prepare for reduced traction

	White
	
	All Stations
	Slow moving vehicle on track
	Prepare for evasive action

	Black
	May have # board
	Starter’s Stand & Black-flag station
	Stewards want to talk to you
	Come into pits ASAP

	Black
	Closed
	Starter’s Stand
	Warning! You are driving in an unsafe manner
	Clean up your act!

	Black w/Orange ball
	May have # board
	Starter’s Stand & Black-flag station
	There is a mechanical problem with your car
	Slow down — 
Come into pits ASAP!

	Checkered
	
	Starter’s Stand
	Race or session is over
	Slow down — 
Complete lap and enter pits


Introduction





C - High Powered Cars








C - Low Powered  Cars





B





APEX





A











Traditional Cornering





1.	Establish A, B and C (landmarks).


2.	Brake hard and downshift from A to B.


3.	At B release brake, turn wheel into corner and apply enough power to induce a drift.


4.	Increase power to full throttle from B to C.


5.	With practice, move A toward B and C toward B.





Braking into the Corner (Trail Braking)





1.	Establish A, B and C (landmarks).


2.	Brake hard and downshift from A to B.


3.	Brake at a reduced rate from B to C.


4.	Accelerate from C.


5.	With practice, move A toward B and C toward B.





NOTE:    Braking into the corner allows later braking and is faster in some corners but requires more skill and better brake balance.  It is especially useful when the car is set up so that brake pedal can be reached with the left foot for simultaneous throttle and brake application between B and C.





NOTE:    Landmarks are extremely important.  It is virtually impossible to drive fast consistently without them.








FIGURE 1





FIGURE 2








2

